A few items to consider about this heater

- You may notice that the heater is slightly angled upward. This is to make sure the infrared waves are not hitting the surface of a wooden deck and to keep surface temperature cooler.

- It is important to keep this heater away from direct wind. The casters on this heater allow the user to turn the front of the heater away from the wind. Windy conditions blow the heat away from the heater anyway and defeat the purpose of the heater.

- It is recommended to turn the propane tank off when not in use.

- When hooking up a propane tank for the first time you must bleed out the air in the gas lines which may make the heater difficult to light on the first try. Run the heater on pilot mode for about 2 minutes prior to starting. Light the first stage (bottom 3 burners) and then after 2 minutes light the top stage burner. This will help break in the heater during its first use.